# KBE assignment proposal

The objective of this assignment is the development of a Knowledge-Based Engineering (KBE) application, using the ParaPy KBE platform, to support the design of an engineering system of your choice or one proposed by a company.

Although relatively simple and limited in scope, your application is expected to exhibit several key characteristics of an industry-grade KBE system. In particular, it will:

- provide a parametric model of the product under consideration;

- embed engineering rules to generate design solutions compliant with best practices, design guidelines, or regulations;

- manage rule violations, for example by issuing warnings, suggesting corrective actions, or automatically resolving violations;

- extract information from the parametric model to feed internal and external analysis modules, tools, or services;

- read input data from external files (e.g. XML, Excel, .dat), while allowing interactive modification of selected inputs within the KBE graphical user interface (either the standard ParaPy development GUI or a custom web-based GUI);

- generate output files (e.g. PDF, XML, Excel, text files) reporting relevant data produced by the application (such as calculation results, bills of materials, or plots);

- export (part of) the generated geometry to external CAD/CAE systems using standard exchange formats such as IGES or STEP.

Once you have acquired sufficient understanding of KBE technology and practical experience with the ParaPy system, you will be able to identify relevant design challenges for which KBE can provide effective solutions. Together with your teammate, you will select a design challenge and formulate a corresponding KBE assignment proposal.

To write and submit your assignment proposal, you must use the template and guidelines provided in Appendix A.

## Need inspiration for a suitable design challenge?

- Examples of past KBE assignments (formulated by the course coordinator) are available on Brightspace. You are strongly encouraged to scale down the scope and complexity of your proposed assignment with respect to those examples, as you are now responsible both for defining the assignment and for performing the required knowledge acquisition (i.e. assembling the informal knowledge model, one of the assignment deliverables).

- Consult the videoclips and slide sets available on Brightspace, showcasing KBE applications developed by students in previous editions of the course.

- Each year, several companies and research institutes propose topics for possible assignments. These offer an opportunity to develop a KBE application of direct relevance to an industrial or research context. Please monitor Brightspace for updates if you wish to pursue this option.

## Proposal review and approval process

Once your assignment proposal is complete and formulated according to the template in Appendix A, you must upload it to Brightspace for review and approval (please allow a couple of days for feedback).

Iterations may be required if the proposal is incomplete, unclear, unsuitable for KBE, or unfeasible within the available time or given the capabilities of the ParaPy platform.

It is strongly advised to start developing code only after proposal approval, in order to avoid major rework based on received feedback.

Important notes

- Assignment proposals that are not compliant with the provided template (Appendix A) will not be reviewed.

- KBE applications that are not based on an approved proposal will not be accepted for examination.

- The same submission, review, and approval process applies whether the assignment proposal is based on your own idea or on a topic suggested by a company.

## Ready to submit your assignment proposal?

Before submitting your proposal, use can use the self-checklist below to make sure all items are satisfied.

A. KBE Justification

☐ The design problem is clearly defined and sufficiently complex
☐ The application supports engineering decisions, not only geometry generation
☐ The justification explains why KBE is suitable, based on problem characteristics
☐ It is clearly explained why:

☐ general-purpose programming alone is insufficient

☐ a parametric CAD model alone is insufficient

B. Rule-Based Parametric Model

☐ Main parameters controlling the design are clearly identified
☐ Multiple engineering rules are described (not only equations)
☐ Rules reflect design guidelines, constraints, or best practices
☐ A strategy to handle rule violations is described

C. Architecture & UML

☐ A correct UML class diagram is provided
☐ The diagram includes:

☐ inheritance

☐ composition/aggregation

☐ multiplicities
☐ Classes represent knowledge and logic, not only geometry
☐ The architecture supports:

☐ internal analysis modules

☐ external tool integration

☐ input/output handling

D. Analysis Capabilities

☐ At least one internal analysis is clearly defined
☐ At least one external analysis tool is identified
☐ The interaction with external tools is explained
☐ Analyses influence geometry, sizing, or decisions

E. Input & Output Handling

☐ At least one external input file is used
☐ Input format(s) are clearly specified
☐ Input validation or checking is considered
☐ At least one output file reports analysis results
☐ Geometry export via STEP or IGES is included

F. Feasibility & Scope

☐ The scope is realistic for a 4 week development period
☐ The required data and rules are accessible
☐ The proposal meets all mandatory course requirements

Final check

☐ The proposal follows the provided template
☐ The document is complete and well structured
☐ The UML diagram is readable and consistent with the text

# Appendix A: KBE assignment proposal template

Use the template below to compose your assignment proposal, also when the proposal is based on a topic suggested by a company.

- Upload your assignment proposal to Brightspace in PDF format for review and approval.

- Name the file according to the following example: Team03_deVries_Jansen.pdf

- Assignment proposals not compliant with the provided template or that are incomplete will be rejected without feedback

| ## Team n. 16 | ## Name | ## st. number | ## Name | ## st. number |
| --- | --- | --- | --- | --- |
|  | ## Lucas Huirne | ## 5680778 | ## Ishaan Desai | ## 5032172 |
| ## Bio-Inspired Heat Exchanger |  |  |  |  |
| ## ☐ Own proposal ☑ Company-suggested topic (MTC ) |  |  |  |  |
| The demand for lightweight, high-performance heat exchangers is rapidly increasing. In aerospace, this is driven in large part by increased attention to sustainability. Legacy heat exchangers constructed using subtractive manufacturing have inherent material waste, and are limited in geometric complexity (therefore, thermal performance). Additive Manufacturing (AM) enables the cost and time efficient generation of complex geometries such as lattice structures and quasi-organic forms. Consequently, AM synergizes well with biomimetic design. As living organisms require efficient heat and fluid transfer mechanism with sufficient strength and low weight, biological structures have potential to inspire next-generation heat exchangers with advanced performance. Of particular interest are Triply Periodic Minimal Surfaces (TPMS), implicitly defined in three dimensions, which can mimic organic structures such as endoplasmic reticulum (optimized for chemical transfer) and bone tissue (optimized for mass). The application outlined here addresses a topic proposed by UK-based RTO, Manufacturing Technology Centre (MTC). MTC has expressed interest in a tool capable of generating and analyzing bio-inspired heat exchange geometries for Laser Powder Bed Fusion (LPBF). The suggested use case is an aircraft electronic component cooling unit. The application will generate parametric heat exchanger internal geometries using ParaPy with TPMS’s. The application will export STEP files for CAD and STL files for direct 3D printing. A 3D CFD analysis using Darcy flow formulations to model the heat exchanger structure will validate the thermal-hydraulic performance of the generated designs. The expected benefits include drastically reduced design iteration time, embedded Design for Additive Manufacturing (DfAM) knowledge, and the ability to rapidly explore the design space of geometries for a given thermal challenge, for multiple designs at the same time. This challenge is ideal for KBE because it involves many interdependent design rules (thermal correlations, fluid mechanics, DfAM constraints) and multi-parameter optimization across several iterations. General purpose programming languages lack built-in parametric modelling, geometry interaction, GUI, and geometry export capabilities that ParaPy provides, and a CFD batch run would fail to assess and optimize different categories of KPIs (Manufacturability, thermal performance, hydraulic performance) simultaneously. Although parametric CAD systems exist, they alone cannot create a design with the performance targets as its input, rather than a design as input. The KBE approach uniquely integrates rule-based design, parametric geometry, optimization and validation tool integration into a single workflow. This workflow consists of sequential and parallel steps that otherwise would each be very complex and time-consuming to create manually. |  |  |  |  |

## Rule based parametric model

The heat exchanger lattice can be represented first by its bounding geometry, which is created by the user in the first step of the KBE workflow based on the space requirements of the real-world application. The lattice interior shape is parametrized by a grid of points geometrically spread in space over the geometry, where each point is associated with three scalars. Three fields are created by interpolating between these points for each of the different components using radial basis interpolation. These fields are kxx,y,z,kyx,y,z,kzx,y,z, which represent the local wavenumber for the Triply Periodic Minimal Surface that parametrises the lattice. For example, a gyroid: sinkxxcoskyy+sinkyycoskzz+sinkzzcoskxx=0 Additionally, the lattice has a wall thickness, which is extruded from this surface. The curve above describes the geometry implicitly. ParaPy is based on OpenCascade, which cannot represent Implicit geometry (we contacted ParaPy about this), so the lattice itself will not be represented in ParaPy until the very end, when it is converted to an (approximate) B-Rep, usable for Boolean operations with the encapsulation, and to be exported to STEP. This is unfortunate, but also the only option, because Bio-Mimetic Heat exchangers structures are inherently complex-shaped, which means that exact CAD representation will always require approximation. The lattice is made of a material represented by a density and volumetric heat capacity. An input for the optimization is the manufacturability constraints, namely the minimum wall thickness, feature size, and maximum overhang angle. The fluid is described using its kinematic viscosity and conductivity. The fluid and the lattice are both part of the interior of the heat exchanger. The heat exchanger has properties on its own as well, because the lattice and fluid form one mesh for the simulation. The lattice-in-mesh porous approximation is parametrized using the Darcy number, which is slowly decreased between iterations (making the lattice less permeable). The encapsulation can be represented by the variables for wall thickness (per wall), inlet and outlet location, inlet and outlet diameter, inlet and outlet tube thickness and length, inlet and outlet flange diameter and thickness, attachment type. Input parameters on the heat exchanger system level are inflow velocity, inflow temperature, exterior temperature, outlet pressure, mechanical dissipation (using a lower and upper bound, as this is a constraint) and wall heat conduction rate. Output Heat Exchanger parameters are the mean temperature, outlet temperature (if this is the parameter set to be minimized, otherwise mechanical dissipation can also be minimized). All parameters can be non-dimensionalized (i.e. to create Nusselt number, Darcy friction factor, Colburn j-factor, inflow Reynolds-number). The first engineering rule that will be used is the semi-empirical rule from https://doi.org/10.1016/j.enconman.2023.116955 that can be used to calculate the solidity (and thereby wall thickness) of the heat exchanger interior, and to extrapolate the turbulent behavior from the laminar simulations. Because of the complex geometry and physics of heat exchangers, in order to say anything useful beyond a simple one-equation correlation, simulation and optimization is needed. Therefore, the next engineering rule is that the components of the radial basis interpolation field should be changed in such a way that either the outlet temperature or the mechanical dissipation is minimized (given the constraints). Most parameters can be changed by the user through menus. The bounding geometry can be created by the user by entering corner coordinates in a window, which will be visualized to the user. The simulations can run automatically in sequence for different parameter values, and the KBE app will suggest new (better) parameters, which can also be automatically simulated. This will create an optimized design.

## Internal analysis/sizing capabilities

The following internal analysis and sizing capabilities will be implemented directly within the ParaPy KBE application: - Semi-empirical solidity sizing As mentioned in the previous section, a semi-empirical rule will be used to calculate the required solidity of the heat exchanger. For example, if the KBE-app is set to mechanical dissipation-minimizing mode (mechanical dissipation is pressure difference times mass flow rate), then the Nusselt number is set as a constraint for certain Reynolds number. The corresponding Nusselt number at laminar conditions is then calculated (which corresponds to the simulation), and the minimum solidity that can achieve that Nusselt number is calculated using the semi-empirical rule. Because this solidity is minimal, this in turn has a minimizing effect on the mechanical dissipation rate. The semi-empirical formulas are also used to extrapolate the laminar simulation to the required turbulent operation condition. - Mass Calculation Given a design, its mass can be calculated by integrating its density field (which is given as gamma to OpenFOAM). This does not drive the design; it is just a number that is displayed during the optimization process for the different designs. - Manufacturability analysis Given a design, a penalty can be calculated for the wall thickness and overhang constraint. Using the methods in https://doi.org/10.1016/j.ijheatmasstransfer.2024.126299. This means that the design (represented with the gamma field) is analysed for thin walls and large overhang angles, which create a manufacturability score / constraint-breaking penalty. This penalty is fed back to the optimizer, which causes the optimizer to create a manufacturable design. Therefore, this internal analysis tool greatly drives the design outcome.

## Link(s) with external analysis/sizing capabilities

- OpenFoam CFD Solver The module includes a 3D CFD capability using Darcy-Brinkman flow formulation to model the heat exchanger interior as a porous medium, enabling fast but physically meaningful validation of the thermal-hydraulic performance without resolving every geometric detail. This is a common method to quickly and accurately assess heat exchangers (and is used in many papers such as https://doi.org/10.1016/j.ijheatmasstransfer.2024.126299 and https://doi.org/10.1016/j.ijheatmasstransfer.2025.127288 ). As the total heat generation from viscosity is not very geometry-dependent, it can be extrapolated from this model that does not resolve boundary layers. Turbulent heat distribution is more geometry-dependent, but it is assumed that this can also be extrapolated from the laminar simulation. This approach keeps the computation fast (the optimization loop runs in approximately half an hour) while capturing the essential flow and heat transfer physics. The CFD results can be exported as OpenFOAM case files for detailed visualization in ParaView. A custom OpenFOAM solver is used that also solves the adjoint flow and heat flow equations, which provide sensitivities such that the optimizer can more efficiently optimize the design. This solver is called MTO_TF (https://github.com/MTopOpt/MTO , https://doi.org/10.1007/s00158-020-02731-z ). - PySLM DfAM final manufacturability analyzer After a geometry is selected (by choosing the correct radial basis interpolation values, and wall thickness) by the user or the optimizer, the KBE app generates a STL file based on the design. This STL is created based on the gamma field that represents the design. This STL is then imported into PySLM (an additive manufacturing analysis package with a python-interface) to analyze the manufacturability with regards to wall thickness, overhang angle and total mass. This is separate from the internal analysis tools mentioned earlier, that perform analyses based on the voxelated gamma field. Rather, PySLM will analyze the final STL design. We understand the great complexity of creating fluid-based topology optimizer. Therefore, we have worked ahead to create the simulation and optimizer backend. This can be tested by accessing https://github.com/HuiLucas/MTO and using docker build to turn the dockerfile into a container. While working ahead is against course recommendations, the requirements of this particular company assignments necessitated it. CFD simulation is the only suitable method to analyze the heat exchanger on a sufficiently accurate level to differentiate between designs, and optimization is required for analysis to drive the design. This is especially true in the case of the complex biomimetic geometries targeted by our application.

## External input data handling capabilities

1. Design requirements file (JSON) This file specifies the top-level design requirements and parameters as explained in the rule-based model section. The JSON format allows structured, human-readable specification of design scenarios. Different use cases (e.g., different MTC projects or aerospace electronic cooling applications) can be loaded by selecting a different JSON file, without modifying the application code. The program can be run completely without GUI interaction using the JSON. This also allows integration of the KBE app with other apps, or to run multiple KBE apps at the same time that each optimize an heat exchanger for different conditions. 2. Material and manufacturing constraints file (Excel .xlsx) This spreadsheet contains material properties (thermal conductivity, density, specific heat capacity) for candidate materials (e.g., SS316L, AlSi10Mg, Ti6Al4V) and DfAM constraints for the target LPBF machine (minimum feature size, maximum build volume, layer thickness, overhang angle limit). Multiple material/machine combinations can be stored as separate rows or sheets. Interactive editability Key parameters from both input files (target outlet temperature, flow rates, material selection, dimensional envelope, and unit cell type) will be exposed in the ParaPy GUI for interactive modification, enabling rapid design exploration without editing the source files. When entering the geometry envelope of the heat exchanger, and during inlet-/outlet-specification, the user will be able to see and interact with the geometry in the GUI, as well as when the final result is generated. The lattice will be shown during every optimization iteration. Input validation The KBE application will validate all input data upon loading: checking for missing or out-of-range values (e.g., negative flow rates, temperatures below absolute zero), verifying unit consistency, and ensuring that the specified dimensional envelope is compatible with the selected LPBF machine’s build volume. Validation errors will be reported to the user with descriptive messages in the GUI. Validation logic is implemented entirely within the KBE application, not in a spreadsheet.

## Output data reporting capabilities

1. STEP file (.stp) or IGES file The primary geometric output, exported with correct units (millimetres), containing the heat exchanger assembly (shell, internal features). This enables downstream use in CAD systems for detailed analysis by MTC or other stakeholders. This must be generated from the STL file by surface approximation of the implicit curves. 2. STL file (.stl) An export of the final implicit geometry (generated by the Implicit Gyroid Modeller) in STL format, ready for slicing and 3D printing. This is the direct manufacturing output of the application. 3. Design summary report (PDF) An automatically generated PDF report containing: all input parameters and design requirements, computed thermal-hydraulic performance (heat transfer rate, pressure drop, effectiveness), mass estimate and comparison to target, DfAM compliance summary (pass/fail for each manufacturing constraint), key geometric dimensions, and visualisations of the heat exchanger geometry. This report is intended for design review and serves as a traceability document linking inputs to outputs. 4. CFD results output (PDF, PNG) A summary of the Darcy flow CFD validation results, including temperature and velocity field visualisations, and pressure drop compared to the correlation-based predictions, and an assessment of the thermal-hydraulic performance. Outputs such as Mechanical Dissipation and outlet temperature are plotted for every design iteration, showing convergence. OpenFOAM case files are also exported for detailed post-processing in ParaView. This output is intended for engineering review and verification purposes.

## Architecture of the KBE application (standard UML class diagram)

Describe the intended architecture of your KBE application by means of a standard UML class diagram. The diagram must illustrate: - the main classes and their responsibilities; - inheritance relationships; - composition/aggregation relationships (with multiplicity); - the ownership of key parameters and engineering rules. Notes: - The purpose of this diagram is to communicate your conceptual architecture and design intent, not to describe implementation details. - Classes should represent engineering knowledge and logic, not only geometric decomposition - The diagram does not need to be complete or final. - It must be sufficiently detailed to assess architectural soundness, feasibility, and suitability for a KBE application. - Low-level implementation aspects (e.g. ParaPy-specific attribute types, execution order) must not be included. - Do not use here the KBE extension for UML, which will be required for final set of assignment deliverables. - Initialization of geometry (set lattice properties, etc.) - Inlet outlet definitions (T, boundary, empirical estimation, etc.) - Running simulation/optimization - Analysis of results (+STL Generation, + export OpenFoam for Paraview) - DfAM analysis (PySLM? +weight analysis) - STEP File generator - Report exporter Parallel - Inputs through xlsx, JSON, yaml etc.